CVA6 PTW Scoreboard Verification Summary

The PTW, or Page Table Walker, is verified as a transaction controller. Unlike the private TLB and shared TLB, the PTW is not mainly a storage structure. It accepts a serialized miss from the shared TLB, requests a Page Table Entry from the data cache, classifies the returned PTE, and then either produces a shared-TLB update, raises an error, raises an access exception, or drains the transaction after a flush.

The compact scoreboard follows the PTW behavior using five abstract states:

SB_IDLE: no active tracked walk.
SB_WAIT_GRANT: a shared-TLB miss was tracked and the PTW should request memory.
SB_WAIT_RVALID: the memory request was granted and the scoreboard waits for returned PTE data.
SB_RETURNED: returned PTE data is now expected to be forwarded if it is a legal leaf PTE.
SB_KILLED: a flush or kill interrupted the walk, so the scoreboard waits until the PTW becomes inactive.

The main liveness property says that under bounded memory fairness, every accepted PTW request must eventually make progress. Progress means one of the following: a valid shared-TLB update, a PTW page fault, a PTW access exception, or the PTW returning inactive after a flush/kill.

The main data-integrity property says that for a successful walk, the PTE returned from the data cache must be the same PTE forwarded to the shared TLB. There are two versions: one for instruction-side misses and one for data-side misses.

For the ITLB version, if the tracked request was instruction-side and the returned PTE is a legal executable leaf, then shared_tlb_update_o.valid must be high, shared_tlb_update_o.content must match the returned PTE, and the update VPN, ASID, VMID, and update virtual address must match the tracked request.

For the DTLB version, if the tracked request was data-side and the returned PTE is a legal readable leaf, and for stores also writable and dirty, then the same forwarding rule must hold.

The “no two misses at the same time” property is not a standalone PTW property.

At PTW level:

The PTW receives only one serialized miss request from the shared TLB.

The scoreboard tracks whether this request came from ITLB or DTLB using itlb_req_i.

At shared-TLB/MMU integration level:

Assert that the shared TLB never forwards both itlb_miss_o and dtlb_miss_o in the same cycle.